Indoor Positioning App

User Guide

# 1. Getting the App

Download the latest version of the APK from the GitHub repository:

GitHub: https://github.com/reetikaagajula/Indoor_Localization_IRIBE/releases/latest

This app is actively maintained. New features, bug fixes, and improvements are regularly added to the GitHub repository. Always download the latest APK from the link above to make sure you have the most up-to-date version.

To install on your Android phone:

- Download app-debug.apk from the GitHub Releases page

- On your phone go to Settings → Security → enable Install from unknown sources

- Open the downloaded APK file and tap Install

- Open the app — tap Allow when it requests Location permission

# 2. Requirements

- Android phone running Android 9.0 or higher

- WiFi enabled

- Location permission set to Allow all the time

- Pixel 7 Pro recommended for best RTT ranging performance

# 3. App Screen

When you open the app you will see:

| Element | What it does |
| --- | --- |
| Landmark Label (text box) | Type the name for your current location before scanning. This becomes the filename of the saved CSV. Example: p1, p2, corridor_5207 |
| Scan Once button | Performs a single WiFi scan and saves the result immediately. |
| Start Continuous button | Automatically scans every 3 seconds. Tap again to stop. |
| Rows / Scans counter | Shows how many data rows and scans have been recorded. |
| RTT capable | Shows how many nearby access points support RTT distance ranging. |
| Status box | Live log showing scan results and confirming when data is saved. |

# 4. How to Use the App

## Collecting data at a location

- Walk to your chosen spot and stand still

- Type a label in the Landmark Label box — for example p1

- Tap Start Continuous

- Stand completely still for 30 to 60 seconds

- Tap Stop Continuous

- Move to the next spot and repeat with a new label — p2, p3, and so on

## Tips

- Always stand still while scanning. Moving reduces data quality.

- Hold the phone in your hand — do not put it in your pocket while scanning.

- Use a different label for every location. Reusing a label appends to the same file.

- Aim for at least 30 different locations spread across the corridor.

# 5. Where Data is Saved

Each label creates a separate CSV file saved on the phone at:

/sdcard/Android/data/com.research.indoorpositioning/files/

For example, typing p1, p2, p3 produces:

p1.csv   p2.csv   p3.csv

To copy all files to your laptop, connect via USB and run:

adb devices

Then use the device ID shown to pull the files:

adb -s DEVICE_ID pull /sdcard/Android/data/com.research.indoorpositioning/files/ C:/Downloads/csv_data/

# 6. Troubleshooting

## RTT capable shows 0

- Make sure WiFi is turned on

- Set Location permission to Allow all the time

- Move closer to an access point

## Files not appearing after ADB pull

- Tap Stop before running the pull command

- Enable USB Debugging in Developer Options on your phone

## App crashes on launch

- Go to Settings → Apps → IndoorPositioning → Permissions → set Location to Allow all the time

- Reinstall the latest APK from GitHub if the problem persists

# 7. Updates & Future Changes

This app is under active development. New features and improvements are continuously being added and will be published to the GitHub repository. Check back regularly for updates.

Planned improvements include:

- Real-time location prediction displayed directly in the app

- Google Drive auto-upload of collected CSV files

- Support for additional building floors

- Improved RTT ranging accuracy and outlier filtering

- Visual floor plan overlay showing your estimated position live

To get the latest version:

- Visit the GitHub repository

- Go to the Releases page and download the newest app-debug.apk

- Install it over the existing app — your data files will not be deleted

GitHub: https://github.com/reetikaagajula/Indoor_Localization_IRIBE/releases/latest